- 16 to 1 Mux

  - Pure Behavioral:

module mux16to1 (in,sel,out);
  input[15:0] in;
  input[3:0] sel;
  output out;
  assign out = in[sel] //non constant index in expression on RHS generates a MUX (during synthesis)
endmodule

module muxtest;
  reg[15:0] A;
  reg[3:0] S;
  wire F;

  mux16to1 myMux(.in(A), .sel(S), .out(F));

  initial
   begin
    $dumpfile("mux16to1.vcd");
    $dumpvars(0,muxtest);
    $monitor($time, "A=%h, S=%h F= %b", A,S,F);
    #5 A=16'h3f0a; S= 4'h0;
    #5 S=4'h1;
    #5 S=4'h6;
    #5 S=4'hc;
    #5 $finish
   end
endmodule

- Using five 4 to 1 Muxes

module mux4to1 (in,sel,out);
  input[3:0] in;
  input[1:0] sel;
  output out;
  assign out = in[sel];
endmodule

//structural description, using behavioral description of 4 to 1 mux
module mux16to1(in,sel,out);
  input[15:0] in;
  input[3:0] sel;

output out;
  wire[3:0] t;

  mux4to1 M1 (.in(in[3:0]),.sel(sel[1:0]), .out(t[0]));
  mux4to1 M2 (.in(in[7:4]),.sel(sel[1:0]), .out(t[1]));
  mux4to1 M3 (.in(in[11:8]),.sel(sel[1:0]), .out(t[2]));
  mux4to1 M4 (.in(in[15:12]),.sel(sel[1:0])
  mux4to1 M5 (.in(t),.sel(sel[3:2]), .out(out));

endmodule

- Use three 2 to 1 Mux to model 4 to 1 Mux

module mux2to1 (in,sel,out);
  input[1:0] in;
  input sel;
  output out;
  assign out = in[sel];
endmodule

module mux4to1(in,sel,out);
  input[3:0] in;
  input[1:0] sel;

output out;
  wire[1:0] t;

  mux4to1 M1 (.in(in[1:0]),.sel(sel[0]), .out(t[0]));
  mux4to1 M2 (.in(in[3:2]),.sel(sel[0]), .out(t[1]));
  mux4to1 M3 (.in(t),.sel(sel[1]), .out(out));

endmodule

- Structural Modelling of 2 to 1 Mux

module mux2to1 (in,sel,out);
  input[1:0] in;
  input sel;
  output out;
  wire t0,t1,t2;

NOT G0 (t0, sel)
  AND G1 (t1,in[0],t0);
  AND G2 (t2, in[1], sel);
  OR G3 (out, t1,t2);
endmodule

- 16-bit Adder

Generation of status flags: sign, zero, carry, parity, overflow

- Pure Behavioral

module ALU (X,Y,Z,Sign,Zero,Carry,Parity,
  input[15:0] X,Y;

output[15:0] Z;
  output Sign,Zero,Carry,Parity,

  assign {Carry,Z} = X + Y;
  assign Sign = Z[15];
  assign Zero = ~|Z;
  assign Parity = ~^Z;
  assign Overflow = (X[15] & Y[15] & ~Z[15]) |
                    (~X[15] & ~Y[15] & Z[15]);
endmodule

module alutest
  reg[15:0] X,Y;
  wire[15:0] Z;
  wire S,ZR,C,P,O;
  initial
    begin
      $dumpfile("alu.vcd");
      $dumpvars(0,alutest);
      $monitor($time, "X=%h, Y=%h, Z=%h, S=%b, ZR=%b, C=%b, P=%b, O=%b", X,Y,Z,S,ZR,C,P,O);    
       #5 X=16'h8fff; Y=16'h8000;
       #5 X=16'hfffe; Y=16'h0002;
       #5 X=16'hAAAA; Y=16'h5555;
       #5 $finish

end
endmodule

- Use Four 4-bit Adders

module ALU (X,Y,Z,Sign,Zero,Carry,Parity,
  input[15:0] X,Y;

output[15:0] Z;
  output Sign,Zero,Carry,Parity,

wire[3:1] c;

  adder4 A0(Z[3:0],c[1],X[3:0],Y[3:0],

adder4 A1(Z[7:4],c[2],X[7:4],Y[7:4],

adder4 A2(Z[11:8],c[3],X[11:8],Y[11:

adder4 A3(Z[15:12],Carry,X[15:12],Y[
  assign Sign = Z[15];
  assign Zero = ~|Z;
  assign Parity = ~^Z;
  assign Overflow = (X[15] & Y[15] & ~Z[15]) |
                    (~X[15] & ~Y[15] & Z[15]);
endmodule

module adder4 (S, cout, A, B, cin);
  input[3:0] A, B;

output[3:0] S;

input cin;

output cout;
  assign {cout,s} = A + B + cin;
  
endmodule

- Model 4-bit Adder as Ripple Carry Adder (4 Full Adders)

module adder4 (S, cout, A, B, cin);
  input[3:0] A, B;

output[3:0] S;

input cin;

output cout;

wire c1,c2,c3;
  fulladder FA0(S[0], c1, A[0], B[0], 1'b0);
  fulladder FA1(S[1], c2, A[1], B[1], c1);

fulladder FA2(S[2], c3, A[2], B[2], c2);

fulladder FA3(S[3], cout, A[3], B[3], c3);  
endmodule

- Model Full Adder using 3 XOR and 2 OR Gates

module fulladder (s,cout,a,b,c);
  input a,b,c;
  output s, cout;
  wire s1,c1,c2;
  xor G1 (s1,a,b), G2(s,s1,c), G3(cout,c2,c1);
  and G4 (c1,a,b), G5(c2,s1,c);
endmodule

- Improvement: Carry Lookahead Adder Instead of Ripple Carry Adder

  - generate carry's in parallel, so gate delay is less (in ripple carry adder delay is due to 4 carry delays)

  - See Digital Design notes

- Mux using procedural block

Need the default case as Verilog is a four value system, for cases involving z/x default case will come into play

- Synchronous Up Down Counter using Procedural Blocks

- N-bit Counter

- More than 1 Clocks

- Using Multiple Edges of the Same Clock

t calculated during negedge, then used during posedge

In above example, 2 operations every clock cyele:

- c changed during rising edge

- that value is used during falling edge

- Ring Counter

Wrong implementation: Blocking statements cause count to become 8'b0, then shift

Correct Implementation: Using non-blocking assignments:

RHS's calculated first (for both assignments)
then, assigned to LHS

Now ring counter rotaion works as expeceted

Correct implementation using blocking assigments:

- Parameterized N-bit Up/Down Counter